# Supplementary material

Label construction can determine measured detector performance on small auditory brainstem response benchmarks

All tables are generated by src/revision.py and src/make_supplement.py in the released repository.

Supplementary Table S1. Interaction statistics of the submitted version, retained for continuity

Subject-clustered bootstrap estimates (bootstrap means) of the label × detector interaction on earndb pooled over carrier frequency, as reported in the originally submitted manuscript, with the condition-level and exact-permutation guards added in revision.

| Interaction | Bootstrap ΔΔ [95% CI] | Holm-adjusted bootstrap p | Condition-level ΔΔ [95% CI] | Drop-one-listener range | Exact permutation p (subject level) |
| --- | --- | --- | --- | --- | --- |
| wnsfmp vs LR-eng | −0.249 [−0.344, −0.138] | 0.005 | — | — | — |
| wnsfmp vs GBDT-eng | −0.219 [−0.340, −0.065] | 0.033 | — | — | — |
| wnsfmp vs MLP-raw | −0.347 [−0.418, −0.268] | 0.003 | −0.321 [−0.413, −0.217] | −0.364 to −0.338 | 0.031 |
| LR-eng vs GBDT-eng | +0.030 [−0.048, 0.126] | 0.545 | — | — | — |
| LR-eng vs MLP-raw | −0.098 [−0.179, −0.022] | 0.033 | −0.071 [−0.181, 0.045] | −0.121 to −0.078 | 0.094 |
| GBDT-eng vs MLP-raw | −0.128 [−0.266, −0.029] | 0.008 | −0.091 [−0.246, 0.052] | −0.170 to −0.082 | 0.094 |

Supplementary Table S2. Expert annotation by participant and carrier frequency (coherence check)

Annotated level range, the lowest level at which the expert recorded a response, the number of expert-present levels, and whether the series is non-monotone (a level judged absent above one judged present).

| Participant | Frequency | Annotated levels (dB peSPL) | n levels | Lowest expert-present level | Expert-present levels | Non-monotone | Single-level dips | Surrogate-present levels |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N1 | 1 kHz | 40-100 | 13 | 85 | 4 | no | 0 | 10 |
| N1 | 4 kHz | 40-100 | 13 | 55 | 8 | yes | 2 | 8 |
| N2 | 1 kHz | 40-100 | 13 | 90 | 2 | yes | 1 | 9 |
| N2 | 4 kHz | 40-100 | 13 | 60 | 8 | yes | 1 | 10 |
| N3 | 1 kHz | 40-100 | 13 | none | 0 | no | 0 | 12 |
| N3 | 4 kHz | 40-100 | 13 | none | 0 | no | 0 | 12 |
| N4 | 1 kHz | 25-100 | 16 | 80 | 4 | yes | 1 | 11 |
| N4 | 4 kHz | 25-100 | 16 | 45 | 12 | no | 0 | 13 |
| N5 | 1 kHz | 25-100 | 16 | 85 | 4 | no | 0 | 13 |
| N5 | 4 kHz | 25-100 | 16 | 30 | 14 | yes | 1 | 14 |
| N6 | 1 kHz | 30-100 | 15 | none | 0 | no | 0 | 14 |
| N6 | 4 kHz | 30-100 | 15 | 40 | 12 | yes | 1 | 12 |
| N7 | 1 kHz | 50-100 | 11 | 95 | 2 | no | 0 | 11 |
| N7 | 4 kHz | 50-100 | 11 | 55 | 9 | yes | 1 | 10 |
| N8 | 1 kHz | 30-100 | 6 | none | 0 | no | 0 | 4 |
| N8 | 4 kHz | 40-100 | 4 | none | 0 | no | 0 | 3 |

7 of 16 series are non-monotone under the expert label.

Supplementary Table S3. Level-offset grid for the annotation mapping

Number of annotations that can match an existing corpus record when the annotation levels are shifted by the stated offset. Within ±10 dB the counts are exact, verified against the corpus record inventory; beyond that range they are upper bounds computed from the annotation levels alone (a shifted level outside the corpus range cannot match any record).

| Offset (dB) | Matched annotations | Basis |
| --- | --- | --- |
| -40 | ≤ 110 | upper bound |
| -35 | ≤ 124 | upper bound |
| -30 | ≤ 138 | upper bound |
| -25 | ≤ 152 | upper bound |
| -20 | ≤ 167 | upper bound |
| -15 | ≤ 182 | upper bound |
| -10 | 184 | exact |
| -5 | 193 | exact |
| +0 | 204 | exact |
| +5 | 186 | exact |
| +10 | 170 | exact |
| +15 | ≤ 154 | upper bound |
| +20 | ≤ 139 | upper bound |

Supplementary Table S4. Evaluation-label-only interactions (training label fixed)

ΔΔ = Δ_a − Δ_b with Δ_d = AUROC(eval expert) − AUROC(eval surrogate) for a fixed training label; plug-in estimates. All intervals include zero: with the training label held fixed, switching only the evaluation label does not reorder the detectors.

Training label fixed: surrogate

| Interaction (evaluation label only) | Plug-in ΔΔ (pooled) |
| --- | --- |
| LR-eng vs wnsfmp | +0.111 |
| LR-eng vs MLP-raw | +0.066 |
| GBDT-eng vs wnsfmp | +0.089 |
| GBDT-eng vs LR-eng | -0.022 |
| GBDT-eng vs MLP-raw | +0.044 |
| MLP-raw vs wnsfmp | +0.046 |

Per-detector Δ (pooled): wnsfmp -0.165; LR-eng -0.053; GBDT-eng -0.075; MLP-raw -0.119

Training label fixed: expert

| Interaction (evaluation label only) | Plug-in ΔΔ (pooled) |
| --- | --- |
| LR-eng vs wnsfmp | +0.401 |
| LR-eng vs MLP-raw | -0.088 |
| GBDT-eng vs wnsfmp | +0.322 |
| GBDT-eng vs LR-eng | -0.079 |
| GBDT-eng vs MLP-raw | -0.167 |
| MLP-raw vs wnsfmp | +0.488 |

Per-detector Δ (pooled): wnsfmp -0.165; LR-eng +0.236; GBDT-eng +0.157; MLP-raw +0.324

Supplementary Table S5. Mendeley per-annotator AUROC

Leave-one-subject-out AUROC of each detector on the Mendeley click corpus when scored against each of the six audiologists individually.

| Detector | Annotator 1 | Annotator 2 | Annotator 3 | Annotator 4 | Annotator 5 | Annotator 6 | Spread | Consensus AUROC [95% CI] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMS-SNR | 0.652 | 0.681 | 0.664 | 0.698 | 0.733 | 0.700 | 0.080 | 0.681 [0.491, 0.852] |
| LR-eng | 0.642 | 0.676 | 0.642 | 0.708 | 0.702 | 0.677 | 0.066 | 0.668 [0.476, 0.837] |
| GBDT-eng | 0.599 | 0.617 | 0.591 | 0.667 | 0.672 | 0.670 | 0.082 | 0.599 [0.431, 0.754] |
| MLP-raw | 0.626 | 0.574 | 0.589 | 0.561 | 0.565 | 0.573 | 0.065 | 0.567 [0.388, 0.722] |

Inter-annotator agreement: median pairwise κ = 0.691 (range 0.518–0.951); 26% of conditions not unanimous; surrogate vs consensus κ = 0.248.

Supplementary Table S6. Factorial cells within each carrier frequency

AUROC of each detector trained under one label and scored against each label, within 1 kHz and within 4 kHz (crossed training and evaluation).

| Detector, training label | 1 kHz vs expert | 1 kHz vs surrogate | 4 kHz vs expert | 4 kHz vs surrogate |
| --- | --- | --- | --- | --- |
| wnsfmp (untrained) | 0.734 | 0.855 | 0.768 | 0.793 |
| LR-eng, trained surrogate | 0.651 | 0.747 | 0.746 | 0.754 |
| LR-eng, trained expert | 0.705 | 0.493 | 0.871 | 0.733 |
| GBDT-eng, trained surrogate | 0.575 | 0.677 | 0.713 | 0.723 |
| GBDT-eng, trained expert | 0.666 | 0.513 | 0.763 | 0.697 |
| MLP-raw, trained surrogate | 0.761 | 0.717 | 0.643 | 0.709 |
| MLP-raw, trained expert | 0.883 | 0.455 | 0.853 | 0.698 |

Supplementary Table S7. Main quantities recomputed with participant N3 removed

Participant N3 is the one whom the expert annotated response-absent at every level at both frequencies. All qualitative conclusions are unchanged.

| Quantity | Full sample | N3 removed |
| --- | --- | --- |
| Surrogate vs expert κ | 0.167 | 0.231 |
| Pooled interaction, wnsfmp vs MLP-raw | −0.350 | -0.344 |